To burn bootloader:

- Connect USBtiny to JP2 so that the wiring comes from the back edge of the board with the power and USB jacks.

- Make sure USBtiny has jumper connected to pass 5V USB power to the board, if not, connect power supply and fully power on board.

- Run “Solidoodle_Bootloader_Upload.cmd”

To burn bootloader and firmware at once (slower):

- Connect USBtiny to JP2 so that the wiring comes from the back edge of the board.

- Make sure USBtiny has jumper connected to pass 5V USB power to the board, if not, connect power supply and fully power on board.

- Run “SolidoodlePress_AT90_full_upload.cmd”

To flash firmware over USB (faster, only if bootloader has been burned):

- Make sure board is connected to power and USB and is fully powered on.

- Place jumper on JP3.

- Short reset pin to ground for 1 second.

- Run “SolidoodlePress_AT90_firmware_upload_USB.cmd”

- Remove jumper from JP3, and reset board again.

NOTE: After flashing firmware using any method, all EEPROM settings will be reset to defaults. Take note of any of these if you want to keep them - you will need to set them again. It is also a good idea to manually reset and save default settings after flashing firmware because they may have been moved in memory, especially if the new firmware has a different number of settings stored in EEPROM or if they are stored in a different order. You can do this by running M502 then M500.